# FTAFP: A Feedthrough-Aware Floorplanner for Hierarchical Design of Large-Scale SoCs

Zirui Li∗ lzr_official@bupt.edu.cn Beijing University of Posts and Telecommunications

Beijing, China

# Zixuan Li

gxydbc@bupt.edu.cn Beijing University of Posts and

Telecommunications Beijing, China

# Kanglin Tian∗

tiankl@bupt.edu.cn Beijing University of Posts and

Telecommunications Beijing, China

Shixiong Kai kaishixiong@huawei.com Noah’s Ark Lab, Huawei Beijing, China

# Jianwang Zhai†

zhaijw@bupt.edu.cn Beijing University of Posts and

Telecommunications Beijing, China

Siyuan Xu xusiyuan520@huawei.com Noah’s Ark Lab, Huawei Shenzhen, China

ABSTRACT

# Bei Yu

byu@cse.cuhk.edu.hk

The Chinese University of Hong Kong Hong Kong SAR

Kang Zhao zhaokang@bupt.edu.cn Beijing University of Posts and Telecommunications

Beijing, China

Floorplanning is a critical step in the physical design of digital inte- grated circuits (ICs). As circuit complexity grows, the hierarchical design paradigm of large-scale systems on chips (SoCs) is gradually emerging, introducing new optimization challenges, particularly with feedthrough. Feedthrough is a through-module connection, yet it would require additional buffers and ports inside the mod- ule for data transmission. Excessive feedthroughs will inevitably hinder the routability within reusable modules, causing conges- tion and timing problems. However, few works have addressed the challenges of feedthrough modeling and optimization.

In this work, we propose FATFP, a feedthrough-aware SoC floor- planner, to address the aforementioned issues. First, an estimation model is proposed to assess feedthroughs required in the floorplan. Then, we introduce a novel topological representation, SCB-Tree, which incorporates slack computation into the CB-Tree. We also develop a two-phase simulated annealing (SA) framework and an automatic optimization cost scheme to enhance performance. Exper- imental results demonstrate that our floorplanner achieves notable optimization in terms of common edge, feedthroughed modules, and feedthrough wirelength over previous work, with only minor trade-offs in total wirelength and runtime.

∗Co-first authors with equal contribution.

†Corresponding author.

Permission to make digital or hard copies of all or part of this work for personal or classroom use is granted without fee provided that copies are not made or distributed for profit or commercial advantage and that copies bear this notice and the full citation on the first page. Copyrights for components of this work owned by others than the author(s) must be honored. Abstracting with credit is permitted. To copy otherwise, or republish, to post on servers or to redistribute to lists, requires prior specific permission and/or a fee. Request permissions from permissions@acm.org.

ASPDAC ’25, January 20–23, 2025, Tokyo, Japan

© 2025 Copyright held by the owner/author(s). Publication rights licensed to ACM. ACM ISBN 979-8-4007-0635-6/25/01. . . $15.00

https://doi.org/10.1145/3658617.3697728

Violating Module

Buffer

Feedthrough Ports

Violating Macro

Figure 1: The violations caused by feedthrough insertion in hierarchical floorplanning of the large-scale SoC.

## KEYWORDS

Electronic Design Automation, Floorplanning, Feedthrough

ACM Reference Format:

Zirui Li, Kanglin Tian, Jianwang Zhai, Zixuan Li, Shixiong Kai, Siyuan Xu, Bei Yu, Kang Zhao. 2025. FTAFP: A Feedthrough-Aware Floorplanner for Hierarchical Design of Large-Scale SoCs . In 30th Asia and South Pacific Design Automation Conference (ASPDAC ’25), January 20–23, 2025, Tokyo, Japan. ACM, New York, NY, USA, 7 pages. https://doi.org/10.1145/3658617. 3697728

## INTRODUCTION

Floorplanning is the initial step in IC physical design and signifi- cantly impacts the quality of subsequent design stages, e.g., place- ment and routing. Given a module list and a netlist, the floorplan- ner determines the shape and position of the modules according to specific strategies to achieve comprehensive optimization, e.g., wirelength, area, timing, congestion, etc. Due to the increasing scale and complexity of SoCs, modern chip design commonly uses hier- archy and modularization concepts, which bring the floorplanning problem to the sub-chip level [1]. Hierarchical design decomposes

complex systems into multiple levels of subsystems that encapsu- late specific functionalities and interact through interfaces. Con- currently, modular design facilitates the encapsulation and reuse of functionalities at each level. These integrated, bottom-up design methods significantly speed up the front-end chip design process. However, they also present new optimization challenges in physi- cal design, particularly in hierarchical floorplanning for large-scale SoCs [2, 3], where feedthrough [4] is a key challenge.

𝐴

𝐷

𝐵

𝐶

𝐹

𝐸

(a)

𝐴

𝐷

𝐶𝐸(𝐴, 𝐵)

𝐵

𝐹

𝐸

𝐶

𝐶𝐸(𝐴, 𝐶)

(b)

In hierarchical SoC design, it is important to consider the com- pleteness, reusability, and interaction of modules at each level. When higher-level modules are traversed by wires, feedthroughs are used to connect the nets. To meet connectivity and timing constraints, additional buffers, ports, registers, etc., often need to be inserted. As shown in Figure 1, directly inserting these ele- ments may cause violations with the original components inside the feedthroughed module, necessitating the reevaluation and ad- justment of design feasibility. Such adjustment may also present challenges to subsequent placement and routing, resulting in degra- dation of timing, area, and other metrics [2]. When there are too many feedthroughs, the modules may need to be redesigned, affect- ing the design and reuse of modules at each hierarchy. Therefore, it is necessary to minimize the generation of feedthrough.

The most direct way to minimize feedthrough demand is to re- duce through-module connections and routing. Existing works, such as BOB-RSMT [5] aim to minimize over-block connection by optimizing RSMT construction during the pre-routing and global routing stages. Although optimization in the later physical design stages such as routing can avoid some feedthrough insertions, the optimization space is limited and still inevitably prolongs the de- sign iterations. To improve design efficiency and optimize quality, feedthrough minimization should be implemented as early as pos- sible in the design cycle. Therefore, it is necessary and urgent to consider feedthrough optimization in hierarchical floorplanning.

After decades of development, floorplanning techniques have ad- vanced significantly and can be broadly categoried into three types: analytical, heuristic, and learning-based methods. Analytical-based methods [6, 7] generally adopt a two-stage framework of global distribution and legalization. Although known for their efficiency, quality, and robustness, analytical methods are limited by the need for differentiable constraint models [8, 9], making complex con- straints and objectives challenging to address. Heuristic methods rely on topological representations, e.g., Sequence Pair (SP) [10, 11], Corner Block List (CBL) [12, 13], B*-Tree [14, 15] and their variants. These methods employ heuristic algorithms to optimize floorplans, leveraging geometric relations in different topologies to address complex constraints. For instance, CB-Tree [15] integrates corner stitching into a B*-Tree to refine neighboring relations. Recently,

Figure 2: Feedthrough example with two nets, i.e, 𝑁1 = 𝐴, 𝐵

{	}

{	}

and 𝑁2 = 𝐴, 𝐶 . (a) Floorplan with feedthroughed module

𝐷, 𝐸, 𝐹 ; (b) Floorplan with no feedthrough in net 𝑁1 and 𝑁2.

constrained in representation and generalization, are also unsuit- able for feedthrough optimization. Considering both efficiency and generality, heuristic-based methods are promising approaches to address the feedthrough challenge. In this work, we propose a feedthrough-aware floorplanner to address feedthrough optimiza- tion at the sub-chip level. Our main contributions are as follows:

•

To achieve fast optimization of feedthrough, We introduce an accurate feedthrough model that estimates the length of feedthrough wires, the number of feedthroughed modules, and the length of the common edge.

•

To address the fixed-outline constraint, we propose a new rep- resentation called SCB-Tree, which performs slack computation by corner stitching while packing modules.

•

To better satisfy the fixed-outline constraints and enhance the search for optimal solutions, we propose a two-phase SA frame- work that leverages slack information.

•

We design an automatic cost evaluation method that more effec- tively normalizes wire length and feedthrough-related metrics based on the input module list and netlist.

## PRELIMINARIES

### Fixed-outline Floorplanning

{ | ≤ ≤ }

Let 𝐵 = 𝑏𝑖 1  𝑖  𝑛 be a set of rectangle modules, each module

𝑏𝑖 has width 𝑤𝑖 and height ℎ𝑖 . Modules can be classified into three categories: hard, soft, and pre-placed. Hard modules have fixed dimensions, while soft modules can adjust their shapes within a fixed area; pre-placed modules are hard modules with preset coor- dinates. The connections among modules are described in netlist

{  | ≤ ≤  }

𝑁 = 𝑁𝑖 1  𝑖  𝑚 , where each net 𝑁𝑖 specifies a set of modules

requiring connectivity.

The fixed-outline floorplanning aims to place all modules with- out overlapping in a rectangular outline 𝑅, with width𝑊0 and height

𝐻0. Given the total modules’ area 𝐴 and a maximum whitespace ratio 𝜎, the width 𝑊0 and the height 𝐻0 are calculated as:

,	,

deep learning [16, 17] and reinforcement learning [18–20] frame-

works have been used to formulate floorplanning problem, achiev- ing promising results. However, these methods struggle to handle large cases and lack generalization across diverse circuit designs.

In hierarchical floorplanning for SoCs, complex objectives like feedthrough optimization should be considered, requiring mod- ules within the same net to be placed as adjacent as possible to minimize feedthrough. However, overlaps in the global distribu- tion stage of analytical methods complicate feedthrough handling, which relies on neighbor information. Learning-based methods,

𝑊0 = (1 + 𝜎)𝐴𝜆, 𝐻0 = (1 + 𝜎)𝐴/𝜆,	(1)

where 𝜆 represents the aspect ratio. On the premise of ensuring the above constraints, the floorplanner aims to optimize metrics (e.g. wirelength, feedthrough, etc.) to obtain the optimal floorplan.

  - Feedthrough           Problem Since our work focuses on the feedthrough optimization problem of hierarchical floorplanning for large-scale SoCs, the relevant terms are briefly explained below.

n1

n2	n3

n4	n5

n6	n7

|  |  | b6 |  |  |
| --- | --- | --- | --- | --- |
| b3 |  |  |  | b2 |
|  | b4 |  |  |  |
| b1 |  |  |  |  |

b6

b2

b1

b4

b3

b7

b5

rt :

tr :

bl :

lb :

(a)

(b)

(c)

→

Figure 3: (a) A CB-Tree example. (b) The corresponding floor- plan (tile plane) before packing subtree 𝑛5 𝑛7. (c) The corre- sponding floorplan(tile plane). All top neighbors of module

𝑏1 can be found by tracing through the red pointers.

(  )

Feedthrough. In hierarchical floorplanning, unlike traditional floorplanning, the through-module nets need to be connected by feedthroughs. As described in Section 1, module-scale feedthrough wires may cause problems such as timing and routing congestion. Moreover, the additional inserted registers and buffers may neces- sitate a redesign of the entire module. Therefore, modules within the same net should be adjacently placed to the fullest extent to reduce feedthrough demand. The estimation of feedthroughs in- volves two metrics: total feedthrough wirelength 𝐹𝑇𝐻𝑤𝑙 and the number of feedthroughed modules 𝐹𝑇𝐻𝑛𝑢𝑚 . Let 𝑓𝑡ℎ𝑤𝑙 𝑁𝑖 and

(  )

𝑓𝑡ℎ𝑛𝑢𝑚 𝑁𝑖 represent these metrics for a net 𝑁𝑖 . For example,

for the two nets shown in Figure 2(a), the feedthrough metrics

( ) +  ( ) + ( )

𝐹𝑇𝐻𝑤𝑙 = 𝑤 𝐸 𝑤 𝐹 ℎ 𝐷 , 𝐹𝑇 𝐻𝑛𝑢𝑚 = 3. The more detailed estimating methods will be described in Section 3.1.

(	)

Common Edge. The common edge is defined as the edge shared by two adjacent modules within the same net, as shown in Figure 2(b). The length of this edge dictates the capacity of a module’s ports. A floorplan featuring more total common edge (𝐶𝐸𝑙𝑒𝑛) suggests a higher interactivity between modules, fulfilling more extensive connectivity requirements. Let 𝑐𝑒𝑙𝑒𝑛 𝐴, 𝐵 denote the common edge length between module 𝐴 and 𝐵. Assume that the coordinates of

the bottom-left and top-right corner of the module are (𝑥𝑏𝑙, 𝑦𝑏𝑙 )

and (𝑥𝑡𝑟, 𝑦𝑡𝑟 ). Then, 𝑐𝑒𝑙𝑒𝑛 (𝐴, 𝐵) can be calculated as:

If 𝐴, 𝐵 adjacent horizontally ,then:

Solution Optimization

Module Selection

General

module  or

Neighbor-dissatisfied Module

General

Perturbation

Perturbation to Reduce

Feedthrough

Boundary Shrinking

Module Selection

Other

Metrics

or  General

Feedthrough Metrics

Net Simplification

Module

Total Cost

Feedthrough Estimation

General Perturbation

Packing

Meet termination conditions?

No

Yes

Meet outline constraint?

Floorplanning Result

No

Yes

Cost Evaluation

Construct a SCB−tree

Initialization

Module Set	Circuit Netlist

non-positive slack Module

Figure 4: Flowchart of FTAFP.

provides many efficient operations to support handling geometric constraints. Neighbor finding is the most commonly used one in this work. Figure 3(c) provides an example of finding all neighbors of the target tile at the given direction.

### 2.4	Slack Computation

The slack of a module refers to the range within which it can move without overlapping with or pushing other modules. Slack computation is widely used in constraint graph-based placement legalization algorithms [6], based on the following observations:

•

•

The x and y coordinates of modules are computed separately. In each dimension, the floorplan is constrained by one or more “critical paths” in corresponding constraint graphs.

•

Any change in the location of a module on the critical path will produce overlaps or increase the span of the floorplan.

Take the horizontal constraint graph 𝐺ℎ as an example. Let module

𝑣𝑖 be represented as vertex 𝑣ℎ𝑖 in 𝐺ℎ. Its horizontal slack (x-slack) is calculated as Equation (3) [21] , where 𝑅(·) and 𝐿(·) denote the furthest right and left positions that · can reach:

𝑐𝑒𝑙𝑒𝑛

If

(𝐴, 𝐵) = min(𝐴.𝑦𝑡𝑟, 𝐵.𝑦𝑡𝑟 )− max(𝐴.𝑦𝑏𝑙

, 𝐵.𝑦𝑏𝑙 );

(2)

𝑠𝑙𝑎𝑐𝑘 (𝑣ℎ𝑖 ) = 𝑅(𝑣ℎ𝑖 )− 𝐿(𝑣ℎ𝑖 ).	(3)

𝐴, 𝐵 adjacent vertically ,then:

𝑐𝑒𝑙𝑒𝑛 (𝐴, 𝐵) = min(𝐴.𝑥𝑡𝑟, 𝐵.𝑥𝑡𝑟 )− max(𝐴.𝑥𝑏𝑙 , 𝐵.𝑥𝑏𝑙 ).

### CB-Tree Representation

+

(	)

A CB-Tree is a B*-Tree integrated with corner stitching [15], a clas- sical data structure for representing non-overlapping rectangular modules in the 2D plane (called tile plane). It can be traversed in depth-first search (DFS) order to locate the corresponding modules. Let 𝑛𝑖 be the corresponding tree node of module 𝑏𝑖 . It has coordi- nates 𝑥𝑖, 𝑦𝑖 and width 𝑤𝑖 and height ℎ𝑖 . The tree structure can directly compute the horizontal coordinate of each module. Let 𝑛𝑗 and 𝑛𝑘 be the left and right child of 𝑛𝑖 , then 𝑥 𝑗 = 𝑥𝑖 𝑤𝑖 and 𝑥𝑘 = 𝑥𝑖 . Each module should be placed in the lowest possible position to determine the vertical coordinate, and then the tile plane should be updated. Figure 3 gives a CB-Tree and the corresponding floorplan. The corner stitching models modules and empty spaces as tiles and links all of them with four pointers. As shown in Figure 3(c), the 𝑟𝑡, 𝑡𝑟, 𝑏𝑙, and 𝑙𝑏 pointers point to the tiles adjacent to the top, right, left, and bottom boundaries, respectively. Corner stitching

## FTAFP FRAMEWORK

We first give an overview of our proposed feedthrough-aware floor- planner (i.e., FTAFP), which includes: a feedthrough estimation model, slack computation with corner stitching, a two-phase SA framework, and an optimization cost scheme. The feedthrough esti- mation model initially simplifies the nets into sub-nets, constructs a Minimum Spanning Tree (MST) for each to estimate feedthrough wirelength, and employs a greedy algorithm to count feedthrough modules. The proposed SCB-Tree integrates slack computation with CB-Tree to better exploit the features of boundary and adjacency awareness. Figure 4 shows the overall flow of FTAFP, which adopts a novelly introduced two-phase SA framework. Each phase includes a distinctive module selection and perturbation process, aiming to shrink the boundary and optimize the solution quality, separately. Besides, a meticulously designed cost scheme is applied.

### Feedthrough Estimation Model

In the actual design process, engineers often connect directly to adjacent modules within the net, using feedthroughs only when it is

ℎ

w𝑙𝑚𝑎𝑛ℎ

ℎ′

𝑓𝑡ℎwI

w	w′

Algorithm 1 Net Simplification Algorithm.

∈

Input: A net cell list 𝑁𝑖 and all modules 𝐵	𝑁

Output: Sub-net list 𝑁𝑖 .𝑆𝑢𝑏𝑛𝑒𝑡𝑠

for 𝐵𝑗 ∈ 𝑁, 𝑗 = 1 → 𝑁𝑖 .𝑠𝑖𝑧𝑒 () do

+	(	)

𝑗

𝑘

for 𝐵𝑘 ∈ 𝑁, 𝑘 = 𝑗 → 𝑁𝑖 .𝑠𝑖𝑧𝑒 () do if 𝐵 .𝑛𝑒𝑖𝑔ℎ𝑏𝑜𝑟𝑠.𝑐𝑜𝑢𝑛𝑡 (𝐵 ) then

Figure 5: An example of net simplification. A net with 6 modules which are clustered into 3 sub-nets.

unavoidable to connect through modules outside the net. Therefore, our feedthrough model is based on the following rules:

•

Modules that are directly adjacent are assumed to be connected and no feedthrough is required.

•

There are no feedthrough requirements arising between mod- ules separated by empty tiles (whitespace).

•

Connections between adjacent modules are transitive within the same net.

Based on these rules, feedthroughs are estimated with the mini- mum wire length and the fewest number of modules traversed to ensure all modules within the net form a connected graph. Our feedthrough modeling method is divided into two parts: net simpli- fication and feedthrough estimation.

Net simplification is to cluster adjacent modules into sub-nets, allowing feedthrough estimation to be performed within these sub-nets rather than between modules. At first, we transform nets into undirected graphs, where edges are only constructed between adjacent modules. The connected components within are defined as sub-nets. Algorithm 1 delineates the process described above and Figure 5 presents an example. We utilize the union-find algorithm to merge each pair of adjacent modules within, meanwhile, computing the length of the common edge between the pair using Equation (2). Through the 𝐹𝑖𝑛𝑑 operation,the net 𝑁𝑖 is simplified into a set of

{	| ≤  ≤	⊂	}

sub-nets 𝑁𝑠𝑢𝑏 = 𝑁𝑠𝑞 1  𝑞  𝑛𝑖, 𝑁𝑠𝑞  𝑁𝑖 .

(	)

For the sub-net 𝑁𝑠𝑞, the coordinates of its central node 𝑥𝑞, 𝑦𝑞

are calculated as the average of the central coordinates of all its modules. Treating these centers as nodes, the shortest feedthrough connections between sub-nets can be found using an MST. Let 𝐸𝑖 be the set of edges in the MST for net 𝑁𝑖 . Consider two sub-nets,

𝐴 and 𝐵, connected in the MST, with the edge weight 𝜔 (𝐴, 𝐵), the weighted distance 𝐷𝑤 (𝐴, 𝐵) between which can be calculated by:

𝐶𝐸𝑙𝑒𝑛 = 𝑐𝑒𝑙𝑒𝑛 𝐵𝑗 .𝐵𝑘 ; Union( Find(𝐵𝑗 ), Find(𝐵𝑘 ) );

∈	→	()

for 𝐵𝑗	𝑁𝑖, 𝑗 = 1	𝑁𝑖 .𝑠𝑖𝑧𝑒	do

∈	→	()

for 𝑁𝑠𝑞	𝑁𝑖 .𝑆𝑢𝑏𝑛𝑒𝑡𝑠, 𝑞 = 1	𝑁𝑖 .𝑆𝑢𝑏𝑛𝑒𝑡𝑠.𝑠𝑖𝑧𝑒	do if Find(𝐵𝑗 == 𝑁𝑠𝑞 .𝑝𝑎𝑟𝑒𝑛𝑡 ) then

𝑁𝑠𝑞 .𝑐𝑒𝑙𝑙𝑠push_back(𝐵 𝑗 );

Update the Outline and coordinate of 𝑁𝑠𝑞;

()

if 𝑁𝑖 .𝑆𝑢𝑏𝑛𝑒𝑡𝑠.𝑠𝑖𝑧𝑒	== 0 then

return 𝑖	( 𝑗 )

𝑁 .𝑆𝑢𝑏𝑛𝑒𝑡𝑠.push_back(New 𝑆𝑢𝑏𝑛𝑒𝑡 𝐵  );

𝑁𝑖 .𝑆𝑢𝑏𝑛𝑒𝑡𝑠.

Algorithm 2 Algorithm to Estimate 𝑓𝑡ℎ𝑛𝑢𝑚 .

Input: Two sub-nets 𝐴, 𝐵

Output: Feedthrough number between 𝐴 and 𝐵 𝑓 𝑡ℎ𝑛𝑢𝑚

Let P equal to the tile contains A’s center point;

𝑟𝑒𝑎𝑐ℎ𝑥 , 𝑟𝑒𝑎𝑐ℎ𝑦, 𝑓𝑡ℎ𝑏𝑙𝑘 = 0;

∗

while 𝑟𝑒𝑎𝑐ℎ𝑥  𝑟𝑒𝑎𝑐ℎ𝑦 == 0 do

𝑑𝑖𝑠𝑡 = 0;

𝑁𝑒𝑖𝑔ℎ𝑏𝑜𝑟𝑠𝑠𝑒𝑎𝑟𝑐ℎ = Connect( 𝑃.𝑛𝑏𝑟 (𝑑𝑖𝑟 [0]), 𝑃.𝑛𝑏𝑟 (𝑑𝑖𝑟 [1]) );

for 𝑛𝑏𝑟𝑛𝑒𝑥𝑡 ∈ 𝑁𝑒𝑖𝑔ℎ𝑏𝑜𝑟𝑠𝑠𝑒𝑎𝑟𝑐ℎ do

𝑝𝑛𝑡 = 𝑛𝑏𝑟𝑛𝑒𝑥𝑡 .𝑝𝑜𝑖𝑛𝑡 (𝑑𝑖𝑟 [0], 𝑑𝑖𝑟 [1]);

𝑑𝑖𝑠𝑡𝑥 = (𝐵.𝑥𝑚𝑖𝑑 −𝑝𝑛𝑡.𝑥)∗𝑑𝑖𝑟 [0], 𝑟𝑒𝑎𝑐ℎ𝑥 = (𝑑𝑖𝑠𝑡𝑥 <= 0);

𝑑𝑖𝑠𝑡𝑦 = (𝐵.𝑦𝑚𝑖𝑑 −𝑝𝑛𝑡.𝑦)∗𝑑𝑖𝑟 [1], 𝑟𝑒𝑎𝑐ℎ𝑦 = (𝑑𝑖𝑠𝑡𝑦 <= 0);

if 𝑑𝑖𝑠𝑡𝑥 ∗!𝑟𝑒𝑎𝑐ℎ𝑥 + 𝑑𝑖𝑠𝑡𝑦∗!𝑟𝑒𝑎𝑐ℎ𝑦 > 𝑑𝑖𝑠𝑡 then

=	;

𝑑𝑖𝑠𝑡 = 𝑑𝑖𝑠𝑡𝑥 ∗!𝑟𝑒𝑎𝑐ℎ𝑥 + 𝑑𝑖𝑠𝑡𝑦∗!𝑟𝑒𝑎𝑐ℎ𝑦;

𝑃	𝑛𝑏𝑟𝑛𝑒𝑥𝑡

if 𝑃.𝑖𝑠𝑆𝑜𝑙𝑖𝑑 () then 𝑓𝑡ℎ𝑛𝑢𝑚 + +;

return 𝑓𝑡ℎ𝑛𝑢𝑚 .

where 𝑒 (𝐴, 𝐵) is the edge of the constructed MST, 𝑤 (𝐴) and ℎ(𝐴) are the side length of a sub-net, 𝑎𝑟𝑒𝑎j (𝐴) is the summary of modules within 𝐴, 𝑎𝑟𝑒𝑎(𝐴) is the area of the sub-net 𝐴’s boundary.

𝜔 (𝐴, 𝐵) = [𝑛𝑏𝑙𝑘 (𝐴)+ 𝑛𝑏𝑙𝑘 (𝐵)] /2,

(4)

In addition to wirelength, the number of feedthrough modules (𝑓𝑡ℎ𝑛𝑢𝑚 ) is a crucial factor in assessing floorplan quality—a bet-

where 𝑤𝑙𝑚𝑎𝑛ℎ

center of

𝐷𝑤 (𝐴, 𝐵) = 𝑤𝑙𝑚𝑎𝑛ℎ (𝐴, 𝐵)× 𝜔 (𝐴, 𝐵),

(𝐴, 𝐵) represent the manhattan distance between the

ter floorplan will have fewer feedthrough modules for the same wirelength. To estimate 𝑓𝑡ℎ𝑛𝑢𝑚 , we propose a greedy detection algorithm based on neighbor searching. The algorithm selects the

( )

𝐴 and 𝐵, 𝑛𝑏𝑙𝑘 𝐴 is the amount of modules in sub-net 𝐴.

(  )

For net 𝑁𝑖 , regardless of the actual route shape, its feedthrough wirelength 𝑓𝑡ℎ𝑤𝑙 𝑁𝑖 can be estimated as the sum of the wirelength of each feedthrough edge within the MST:

tile with the largest horizontal or vertical span along the estimated route until it lies within the target sub-net boundary in both direc- tions, as summarized in Algorithm 2. The search begins at the tile containing the center of sub-net 𝐴, using the array 𝑑𝑖𝑟 to indicate

[ ]

the relative 2D direction from 𝐴 to 𝐵 (where 𝑑𝑖𝑟 [0] values −1 and

𝑓𝑡ℎ𝑤𝑙 (𝑁𝑖 ) =

.	.

𝑒 (𝐴,𝐵 )∈𝐸𝑖

𝑓𝑡ℎ𝑤𝑙 (𝐴, 𝐵)× 𝜔 (𝐴, 𝐵),

1 denote left and right, and 𝑑𝑖𝑟 1 values represent up and down). The algorithm finds the neighbor tile with the furthest-reaching

𝑓𝑡ℎ𝑤𝑙 (𝐴, 𝐵) = 𝑤𝑙𝑚𝑎𝑛ℎ (𝐴, 𝐵)− 𝑤𝑙j(𝐴)− 𝑤𝑙j(𝐵) × 𝜔 (𝐴, 𝐵),

𝑤𝑙j(𝐴) =𝑤 (𝐴)+ ℎ(𝐴) × ,𝑎𝑟𝑒𝑎j (𝐴) ,

(5)

endpoint, then checks if the target range is met. If not, it moves to this neighbor tile and repeats until the number of feedthrough mod- ules along each edge 𝑒 is counted, yielding the total feedthrough

2	𝑎𝑟𝑒𝑎(𝐴)

modules for net 𝑁𝑖 .

y-slack

| b5 x:14.7% y:0% |  |  | b7 x:14.7% y:0% |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | b6 x:14.7% y:14% |  |  |  |  |  |
| b3 x:9.3% y:0% |  | b4 x:9.3% y:0% |  |  | b2 x:0% y:0% |  |
| b1 x:0% y:0% |  |  |  |  |  |  |

| b5 x:4.5% y:0% | b6 x:4.5% y:0% |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | b7 x:4.5% y:11.4% |  |  |
| b3 x:-10.5% y:0% |  | b4 x:-10.5% y:4% |  |  |  |
|  |  |  | b2 x:-10.5% y:11.4% |  |  |
| b1 x:-10.5% y:0% |  |  |  |  |  |

| A |  | B | C |
| --- | --- | --- | --- |
| b3 |  |  |  |
| x:33.6% y:36.3% |  | x-slack | b2 x:0% |
| x:0% | b1 |  |  |
| y:36.3% |  |  | y:36.3% |

| b5 x:51.3% y:0% | y- |  |  | lack x- | slack |
| --- | --- | --- | --- | --- | --- |
|  | b6 x:51.3% y:14% |  |  |  |  |
| b3 x:9.3% y:0% |  | b4 x:9.3% y:14% |  |  | b2 x:0% y:36.3% |
| b1 x:0% y:0% |  |  |  |  |  |

| b5 x:4.5% y:0% | b6 x:4.5% y:0% |  | b7 x:4.5% y:0% |  |  |
| --- | --- | --- | --- | --- | --- |
| b3 x:0% y:0% |  | b4 x:0% y:4% |  |  |  |
|  |  |  | b2 x:0% y:0% |  |  |
| b1 x:0% y:0% |  |  |  |  |  |

s

(a)

(b)

(c)

(a)

(b)

Figure 6: (a) An example tile plane contains seven packed modules. (b) Slacks of 𝑏3 are initialized its slacks based on the associated tiles A and B. (c) When packing module 𝑏6, we update module 𝑏5’s x-slack and module 𝑏4’s y-slack as 𝑏6’s.

Packing

Slack Computation

No

Need to update?

Finish packing?

No

Yes

Yes

Update slack by neighbor search

Update the tile plane

Finish

Perturb the SCB−Tree

Detect relevant modules

Initial slack value

Locate a module

Figure 7: The slack computation flow by SCB-Tree.

### Slack Computation by Corner Stitching

To enhance the outline awareness of the CB-Tree while accommo- dating the newly proposed optimization objective, we propose the SCB-Tree, which extends the slack computation to corner stitching. The corresponding floorplan of a given SCB-Tree is determined by traversing the tree in DFS order. Each module’s coordinate is determined based on the principles outlined in Section 2.3. As each module is packed, the tile plane and the slack of each module as- sociated with the current module are updated. After packing a module, the current tile plane appears as shown in Figure 6(a), where tiles with solid lines represent modules and tiles with dashed lines represent empty tiles.

We first set the module’s initial slack to facilitate later updates.

Figure 8: (a) A floorplan with 7 modules, the black arrow represents the critical path on x-dimension, and the x-slack on this path is non-positive. (b) After rotating module 𝑏2 on the critical path, the x-span of the floorplan is reduced.

the packing strategy of the CB-Tree, while initializing the slack. After completing the module packing, we identify adjacent modules through neighbor finding in the corner stitching and update their x-slack and y-slack accordingly.

### Two-phase SA Framework

In traditional SA processes, it is often unclear which modules influ- ence the span of the floorplan, and random perturbation operations fail to effectively control the process of satisfying fixed-outline constraints. Especially when optimizing feedthroughs, frequent changes in neighboring modules often lead to violations of the fixed-outline constraints. In response to the above observations and facts, we propose a two-phase SA framework as shown in Figure 4. The two phases adopt different module-selecting strategies and perturbation operations. In the boundary shrinking phase, modules are classified into two categories: non-positive slack and normal modules by computing their slack. The annealing perturbation op- eration actively selects the non-positive slack modules to better satisfy the fixed-outline constraints. In the solution optimization phase, we propose a novel perturbation that operates on neighbor- dissatisfied modules to reduce feedthroughs.

A critical path is defined as the longest path in a floorplan, and modules on it are non-positive slack modules. There may be mul- tiple critical paths in a floorplan, with a module possibly located on more than one critical path. The length of critical paths is equal to the span of the floorplan. Therefore, the perturbation on non- positive slack modules may crucially impact the outline span. Fig- ure 8 illustrates how relocating modules on the critical path can reduce the floorplan span.

For an unplaced module 𝑏𝑖

with height ℎ𝑏𝑖

and width 𝑤

𝑏𝑖

, the initial

In the first phase of our framework, modules with non-positive

slack in any dimension are more likely to be chosen for pertur-

slack in its x-dimension is the sum of the widths of the empty tiles to its right whose height exceeds ℎ𝑏𝑖 . Similarly, the initial slack in its y-dimension is the height of the empty tile above it whose width exceeds 𝑤𝑏𝑖 . Figure 6(b) gives an example of slack initialization.

Next, we implement the following update strategy to dynamically

adjust the module’s slack during packing. Once a module is packed, the slacks in the x and y dimensions are updated separately based on the corresponding modules to its left and bottom, as shown in Figure 6(c). Since 𝑏1, 𝑏3, 𝑏5 are modules on a critical path and their y-slack is 0, the y-slack of 𝑏1 and 𝑏3 do not need updating. Only the y-slack of 𝑏4 is updated to match that of 𝑏6.

Figure 7 shows our slack computation flow. For a perturbed SCB-Tree, we first determine the positions of modules based on

bation operations. The B*-Tree is commonly used for three main operations, which are explained in the following examples:

•

Op1: Rotate a module. A module is an ideal candidate for this operation if it has a non-positive slack in one dimension but a large slack in another, as shown in Figure 8.

•

•

Op2: Relocate a module. This operation tends to operate on modules with two dimensions of non-positive slack and relocate it to be a child of the module that has a larger slack than its shape. That is, we attempt to move it into a large whitespace. Op3: Swap two modules. This operation is ideal for combining two modules with two dimensions of non-positive slack and one dimension of non-positive slack.

Once the fixed-outline constraints are satisfied, the annealing process enters the solution optimization phase. It mainly targets modules that do not expand the floorplan span for perturbation operations. Area and wirelength optimization are achieved by the above perturbation operations, while for feedthrough optimization, a new perturbation operation is proposed:

•

Op4: Change the module’s neighbors. Select a module with minimal neighbor satisfaction and randomly swap it to its neighborhood-demanding child nodes.

In a floorplan, we analyze each module’s neighbor satisfaction during feedthrough calculation and compare it to its neighbor re- quirement, which is determined by the netlist indicating ideal neigh- boring modules.

### Cost Evaluation

The optimization objective comprises five metrics: area, wirelength, common edge length, feedthrough wirelength, and the number of feedthroughed modules. The cost of the floorplan is calculated by summing weighted metrics. However, the wide range in size and unclear relationship between measurements necessitate additional pre-floorplanning iterations to estimate appropriate weights. To streamline this process, we develop a normalizing method to predict the expectations of each metric.

It’s straightforward to prove that the average Manhattan distance

between every 2 modules in a 𝑛 × 𝑛 square grid is 2(𝑛2 −1) . If we

## EVALUATION

FTAFP is implemented in C++ and operates in single-threaded mode on a Linux system, with Intel(R) Xeon(R) Silver 4214R @ 2.4GHz and 128GB memory. The FTAFP framework is compared with three competitive heuristic-based methods based on the topological rep- resentation: Corblivar [13], SP-FOFP [11], and CB-Tree [15]. Where CB-Tree is reproduced by ourselves, SP-FOFP is an open executable and Corblivar is open source. It should be noted that the goal of our work is feedthrough optimization, which has not been consid- ered in previous works. Therefore, we chose the publicly available floorplaners mentioned above as our baselines. The test cases are derived from the GSRC [22] and MCNC [23] benchmarks. To im- prove the reliability of the results and reduce the impact of random errors from the heuristic algorithm, each benchmark is executed 10 times using our approach and baselines to ensure fair and precise measurements. The results are averaged in the same environment.

  - Results without Feedthrough Optimization First, to verify the effectiveness of the SCB-Tree and optimization framework, we test the performance of the general fixed-outline floorplanning using FTAFP without feedthrough optimization. The evaluation metrics used in this experiment are the half-perimeter wirelength (HPWL), aspect ratio (AR), and CPU runtime (RT). Sim- ilar fixed-outline and AR constraints have been added to all base-

lines for a fair comparison. As shown inTable 1, FTAFP has reduced HPWL compared to all baselines under the fixed-outline constraint

√−

divide a chip consisting of 𝑛 modules into a √𝑛

square grid, each grid’s width and height is

3𝑛 2

× 𝑛 uniform

and strictly meets the aspect ratio constraint. Our FTATP results in average wirelength reductions of 23%, 12%, and 6% over [13],

𝑆. Therefore, under

ideal even partitioning, the total half-perimeter wirelength (HPWL) expectation of 𝑚 nets can be calculated as:

2𝑚(𝑛 − 1)

[11] and [15], respectively. This improvement can be attributed to the combination of slack computation and corner stitching. The proposed SCB-Tree makes full use of the whitespace, resulting in a

𝐻𝑃𝑊 𝐿 =

3√𝑛 − 2

× 𝑆.	(6)

more compact overall floorplan and thereby reducing wirelength.

Due to the slack computation when packing the module, there is a

( − )

Similarly, considering the net 𝑁𝑖 may have 0 to 𝑛𝑖 1 feedthrough paths, the expectation number of feedthroughed modules 𝐹𝑇𝐻𝑛𝑢𝑚 and the feedthrough wirelength 𝐹𝑇𝐻𝑤𝑙 can be computed as:

𝑚

3√𝑛 − 2

𝑖=1

2

(7)

𝐹𝑇𝐻𝑛𝑢𝑚 = 2(𝑛 − 1) ×  𝑛𝑖 − 1 ,

slight increase in the runtime of around 13% compared to [15].

the feedthrough optimization is added. The results are illustrated in

Results with Feedthrough Optimization As the main optimization goal of this work, we test the performance of the proposed feedthrough optimization method. The constraints and settings are the same as in the previous subsection, except that

𝐹𝑇𝐻𝑤𝑙 = 𝐹𝑇𝐻𝑛𝑢𝑚 × 𝑆.

Assuming that 𝑁𝑖 is evenly partitioned and that each module is

Table 2. We developed an evaluator to analyze feedthrough-related metrics for the baselines by parsing their floorplan results and computing all metrics using the same model as our floorplanner.

closely spaced, the parameter 𝑟𝑖 represents the largest perfect square

root that is less than or equal to 𝑛𝑖 . The expectation value of com- mon edge length 𝐶𝐸𝑙𝑒𝑛 can be estimated using:

𝑚

Our method demonstrates significant improvements in feedthrou- gh optimization compared to the three baselines. Compared to the CB-Tree, the FTNUM and FTWL metrics are reduced by 12% and 28% on average respectively, and CEL is also significantly improved

1

𝐶𝐸	=	2(𝑛𝑖 − 𝑟𝑖 )− (𝑛𝑖 > 𝑟 2)− (𝑛𝑖 > 𝑟 2 + 𝑟𝑖 ).	(8)

by 25%. HPWL increased by only 4%, which is considered acceptable

𝑙𝑒𝑛

2 𝑖=1

𝑖	𝑖

in the hierarchical design of large-scale SoC. Additionally, Table 2 demonstrates that our methods excel particularly in cases with

Finally, we can obtain the total cost 𝜙, defined as:

𝜙 = 𝛼 𝐴𝑟𝑒𝑎𝑡𝑜𝑡𝑎𝑙 + 𝛽 𝐻𝑃𝑊 𝐿 − 𝛾 𝐶𝐸𝑙𝑒𝑛 + 𝛿  𝐹𝑇𝐻𝑛𝑢𝑚 + 𝐹𝑇𝐻𝑤𝑙  ,

more multi-module nets(e.g. ami33 and ami49) and a large number of modules(e.g. n100 and n200). For the former, the reason lies in our net simplification which encourages more adjacent arrangements

𝐴𝑟𝑒𝑎

𝐻𝑃𝑊 𝐿

𝐶𝐸𝑙𝑒𝑛

𝐹𝑇𝐻𝑛𝑢𝑚

𝐹𝑇𝐻𝑤𝑙

(9)

of modules in the same net. Larger-scale designs tend to have more feedthrough wires with longer spans and traveling through more

where 𝛼, 𝛽, 𝛾, 𝛿 are the weight of each metric, and 𝐴𝑟𝑒𝑎𝑡𝑜𝑡𝑎𝑙 is the sum of all modules’ area.

modules. Consequently, our feedthrough optimization proves to be more effective in these scenarios.

Benchmarks

Corblivar [13]

SP-FOFP [11]

CB-Tree [15]

FTAFP

Table 1: Comparison of baselines with FTAFP, without feedthrough optimization.

| Case | # Modules | # Nets | HPWL | AR | RT | HPWL | AR | RT | HPWL | AR | RT | HPWL | AR | RT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n10 n30 n50 n100 n200 ami33 ami49 | 10 30 50 100 200 33 49 | 118 349 485 885 1585 123 408 | 47,899 175,684 208,356 328,202 607,503 99,355 1,202,310 | 1.02 0.98 0.74 0.90 0.86 1.08 0.99 | 0.05 0.43 1.22 6.13 33.04 0.48 0.96 | 43,071 160,774 193,478 304,279 554,992 91,037 1,010,759 | 1.00 1.00 1.00 1.00 1.00 1.00 1.00 | 0.1 0.85 2.69 7.91 31.71 0.97 2.06 | 42,007 126,952 165,783 310,582 546,521 96,166 1,042,454 | 0.99 0.98 0.99 1.00 1.00 1.01 1.01 | 3.21 6.51 8.12 21.19 42.80 6.69 7.50 | 40,778 111,877 162,011 293,497 524,989 90,503 1,007,618 | 1.01 1.00 1.00 1.00 1.00 1.00 1.00 | 3.25 7.90 10.25 24.31 52.01 7.16 8.46 |
| Ratio |  |  | 1.23 | 0.94 | 0.18 | 1.12 | 1.00 | 0.24 | 1.06 | 0.99 | 0.87 | 1.00 | 1.00 | 1.00 |

Table 2: Comparison of HPWL, common edge length (CEL), feedthrough number (FTNUM) and feedthrough wirelength (FTWL).

| Case |  | Corblivar [13] |  |  | SP-FOFP [11] |  |  |  | CB-Tree [15] |  |  |  | FTAFP |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | CEL | FTNUM | FTWL | HPWL | CEL | FTNUM | FTWL | HPWL | CEL | FTNUM | FTWL | HPWL | CEL | FTNUM | FTWL |
| n10 | 47,899 | 4,369 | 149 | 15,440 | 43,701 | 4,489 | 146 | 14,098 | 42,007 | 4,619 | 141 | 11,427 | 43,625 | 4,934 | 138 | 10,935 |
| n30 | 175,684 | 1,698 | 483 | 52,325 | 160,774 | 2,363 | 478 | 46,433 | 126,952 | 2,142 | 467 | 47,742 | 142,507 | 3,143 | 454 | 43,181 |
| n50 | 208,356 | 2,436 | 798 | 110,124 | 193,478 | 1,645 | 807 | 102,863 | 165,783 | 2,186 | 776 | 109,105 | 182,524 | 4,141 | 744 | 91,213 |
| n100 | 328,202 | 2,359 | 1,474 | 179,963 | 304,279 | 3,339 | 1,781 | 146,137 | 310,582 | 2,052 | 1,461 | 176,137 | 304,584 | 6,545 | 1,358 | 134,075 |
| n200 | 607,503 | 1,345 | 2,971 | 409,287 | 554,992 | 4,012 | 3,442 | 306,281 | 546,521 | 4,263 | 3,672 | 386,471 | 549,286 | 6,053 | 2,802 | 294,796 |
| ami33 | 99,355 | 45,822 | 172 | 45,810 | 91,037 | 40,376 | 194 | 62,531 | 96,166 | 58,730 | 186 | 51,495 | 93,674 | 67,886 | 152 | 29,848 |
| ami49 | 1,202,310 | 39,634 | 824 | 2,149,060 | 1,010,759 | 105,112 | 722 | 1,180,280 | 1,042,454 | 139,426 | 662 | 1,089,270 | 1,047,228 | 177,436 | 597 | 836,962 |
| Ratio | 1.13 | 0.52 | 1.13 | 1.55 | 1.02 | 0.65 | 1.15 | 1.34 | 0.96 | 0.75 | 1.12 | 1.28 | 1.00 | 1.00 | 1.00 | 1.00 |

## CONCLUSION

We propose a feedthrough-aware floorplanner named FTAFP to solve the feedthrough challenge faced by the hierarchical design of large-scale SoCs. To the best of our knowledge, we are the first to model and optimize the feedthrough problem in floorplanning. We introduce SCB-Tree to better satisfy the fixed-outline constraint and optimization objectives and propose a two-phase SA frame- work with targeted perturbation operations. Experimental results demonstrate that FTAFP can significantly optimize objectives such as feedthrough wirelength, quantity, and common edge resources, thereby better meeting actual design needs.

## ACKNOWLEDGMENT

This work is supported by the National Key R&D Program of China (2022YFB2901100), the National Natural Science Foundation of China (No. 62404021), the Beijing Natural Science Foundation (No. 4244107, QY24216), the MIND project (MINDXZ202404), and AI

Chip Center for Emerging Smart Systems (ACCESS), Hong Kong.

## REFERENCES

- S. Garg and N. K. Shukla, “A study of floorplanning challenges and analysis of macro placement approaches in physical aware synthesis,” International Journal of Hybrid Information Technology, vol. 9, no. 1, pp. 279–290, 2016.

- I.-L. Tseng, “Challenges in floorplanning and macro placement for modern SoCs,” in ACM International Symposium on Physical Design (ISPD), 2024, p. 71–72.

- J. Shin, J. Darringer, G. Luo, M. Aharoni, A. Lvov, G.-J. Nam, and M. Healy, “Floorplanning challenges in early chip planning,” in IEEE International System- on-Chip Conference (SOCC), 2011, pp. 388–393.

- Y. Hong, C. Huang, Y. Gao, and C. Li, “Channel based soc feedthrough inser- tion methodology,” in International Conference on Communications, Circuits and Systems (ICCCAS), 2022, pp. 125–130.

- Y. Zhang, A. Chakraborty, S. Chowdhury, and D. Z. Pan, “Reclaiming over-the-IP- block routing resources with buffering-aware rectilinear Steiner minimum tree construction,” in IEEE/ACM International Conference on Computer-Aided Design (ICCAD), 2012, pp. 137–143.

- F. Huang, D. Liu, X. Li, B. Yu, and W. Zhu, “Handling orientation and aspect

ratio of modules in electrostatics-based large scale fixed-outline floorplanning,” in IEEE/ACM International Conference on Computer-Aided Design (ICCAD), 2023,

pp. 1–9.

- X. Li, K. Peng, F. Huang, and W. Zhu, “PeF: Poisson’s equation-based large- scale fixed-outline floorplanning,” IEEE Transactions on Computer-Aided Design of Integrated Circuits and Systems (TCAD), vol. 42, no. 6, pp. 2002–2015, 2023.

- M. Kuwano and Y. Takashima, “Stable-lse based analytical placement with overlap removable length,” in Workshop on Synthesis And System Integration of Mixed Information technologies (SASIMI), 2010, pp. 115–120.

- A. B. Kahng and Q. Wang, “Implementation and extensibility of an analytic placer,” in ACM International Symposium on Physical Design (ISPD), 2004, pp. 18–25.

- H. Murata, K. Fujiyoshi, S. Nakatake, and Y. Kajitani, “VLSI module placement based on rectangle-packing by the sequence-pair,” IEEE Transactions on Computer- Aided Design of Integrated Circuits and Systems (TCAD), vol. 15, no. 12, pp. 1518– 1524, 1996.

- Q. Xu, S. Chen, and B. Li, “Combining the ant system algorithm and simulated annealing for 3D/2D fixed-outline floorplanning,” Applied Soft Computing, vol. 40, no. C, p. 150–160, 2016.

- X. Hong, G. Huang, Y. Cai, J. Gu, S. Dong, C.-K. Cheng, and J. Gu, “Corner block list: An effective and efficient topological representation of non-slicing floorplan,” in IEEE/ACM International Conference on Computer-Aided Design (ICCAD), 2000,

pp. 8–12.

- J. Knechtel, E. F. Y. Young, and J. Lienig, “Structural planning of 3D-IC intercon- nects by block alignment,” in IEEE/ACM Asia and South Pacific Design Automation Conference (ASPDAC), 2014, pp. 53–60.

- Y.-C. Chang, Y.-W. Chang, G.-M. Wu, and S.-W. Wu, “B*-Trees: a new represen- tation for non-slicing floorplans,” in ACM/IEEE Design Automation Conference (DAC), 2000, p. 458–463.

- H.-F. Tsao, P.-Y. Chou, S.-L. Huang, Y.-W. Chang, M. P.-H. Lin, D.-P. Chen, and

D. Liu, “A corner stitching compliant B*-tree representation and its applications to analog placement,” in IEEE/ACM International Conference on Computer-Aided Design (ICCAD), 2011, pp. 507–511.

- Y. Liu, Z. Ju, Z. Li, M. Dong, H. Zhou, J. Wang, F. Yang, X. Zeng, and L. Shang, “GraphPlanner: Floorplanning with graph neural network,” ACM Transactions on Design Automation of Electronic Systems (TODAES), vol. 28, no. 2, 2022.

- Y. Liu, H. Zhou, J. Wang, F. Yang, X. Zeng, and L. Shang, “Hierarchical graph learning-based floorplanning with dirichlet boundary conditions,” IEEE Transac- tions on Very Large Scale Integration Systems (TVLSI), vol. 32, no. 5, pp. 810–822, 2024.

- Z. He, Y. Ma, L. Zhang, P. Liao, N. Wong, B. Yu, and M. D. Wong, “Learn to floorplan through acquisition of effective local search heuristics,” in IEEE International Conference on Computer Design (ICCD), 2020, pp. 324–331.

- Q. Xu, H. Geng, S. Chen, B. Yuan, C. Zhuo, Y. Kang, and X. Wen, “GoodFloorplan: Graph convolutional network and reinforcement learning-based floorplanning,”

IEEE Transactions on Computer-Aided Design of Integrated Circuits and Systems (TCAD), vol. 41, no. 10, pp. 3492–3502, 2021.

- M. Amini, Z. Zhang, S. Penmetsa, Y. Zhang, J. Hao, and W. Liu, “Generalizable floorplanner through corner block list representation and hypergraph embedding,” in ACM International Conference on Knowledge Discovery and Data Mining (KDD), 2022, pp. 2692–2702.

- J. Cong and M. Xie, “A robust mixed-size legalization and detailed placement algorithm,” IEEE Transactions on Computer-Aided Design of Integrated Circuits and Systems (TCAD), vol. 27, no. 8, pp. 1349–1362, 2008.

- W. Dai, L. Wu, and S. Zhang. (2000) GSRC benchmarks. [Online]. Available: http://vlsicad.eecs.umich.edu/BK/GSRCbench/

- M. C. of North Carolina (MCNC). (2000) MCNC benchmarks. [Online]. Available: http://vlsicad.eecs.umich.edu/BK/MCNCbench/